# Field4D Sensor Status Reporting Pipeline

From device discovery and Firestore metadata to Packet Loss, Last Seen, battery status, permissions, and email delivery

Core principle Firestore determines what should be active. BigQuery determines what actually transmitted.

F4D_mac_to_device  →  Active experiments  →  Current sensors  →  Sensor-specific window  →  25-hour battery query  →  Metrics  →  Permissions  →  Email reports

| Rule | Decision |
| --- | --- |
| Active experiment | active_count > 0 |
| Current sensor | Is_Active = true AND Is_Valid = true |
| BigQuery source | Variable = 'battery', last 25 hours |
| Packet interval | One unique packet every 3 minutes |
| Current transmission | Firestore Last_Seen is 15 minutes old or less |
| Low battery threshold | Battery value < 2700 mV |
| Stopped experiment | Not active in Firestore, but last BQ data is within 24 hours |

Phase A - System Inventory and Firestore Discovery Build the complete list of devices, active experiments, current sensors, and replacement history.

1. Set one fixed report time

What we do: Create a single report_now timestamp at the start of the run and reuse it in every calculation.

What we get: One consistent end time for all windows and comparisons.

Why it matters: It prevents small timing differences between the Firestore, BigQuery, Packet Loss, Last Seen, and battery calculations.

2. Load all devices from F4D_mac_to_device

What we do: Query iucc-f4d.Field4D.F4D_mac_to_device and retrieve every unique Owner and Mac_Address. Also keep Device_Name.

What we get: A complete device inventory containing Owner, Mac_Address, and Device_Name.

Why it matters: This is the starting point and does not depend on whether a device transmitted recently.

Implementation note: Display devices as Device_Name (Mac_Address), for example: Agroforestry ARO (88a29e9c3bfb).

3. Retrieve experiments for every device

What we do: For each Owner + Mac_Address pair, call GET /GCP-FS/metadata/experiments.

What we get: All experiments known in Firestore for the device, including exp_name, active_count, replaced_count, and exp_started_at.

Why it matters: This provides the Firestore view of which experiments exist and which ones should currently be active.

4. Create the active experiments list

What we do: Keep only experiments where active_count > 0.

What we get: One active-experiment record containing Owner, Mac_Address, Device_Name, exp_name, active_count, replaced_count, and exp_started_at.

Why it matters: An experiment remains visible even when it has no recent BigQuery data.

Implementation note: Do not use exp_status to decide whether the experiment is active.

5. Retrieve sensor metadata and Last_Package

What we do: For every active experiment, call GET /GCP-FS/last-package with Owner, Mac_Address, and exp_name. Keep the complete response temporarily.

What we get: The full Firestore sensor list, including LLA, Location, Is_Active, Is_Valid, Last_Seen, Last_Package, and Updated_At.

Why it matters: The complete response is needed both to identify replacements and to create the current sensor list.

6. Identify replaced sensors

What we do: When replaced_count > 0, find records whose Location ends with '-replaced'. Remove the suffix to obtain the base Location and use the old record's Updated_At as the replacement time.

What we get: A replacement map: Base Location → latest replacement Updated_At.

Why it matters: It identifies when the current sensor at that location was installed, so it is not penalized for earlier missing packets.

Implementation note: If several old records exist for the same base Location, keep the latest Updated_At.

7. Create the current active and valid sensor list

What we do: After replacement processing, keep sensors where Is_Active = true and Is_Valid = true.

What we get: The current sensor inventory used for all sensor-level calculations.

Why it matters: Only sensors that are currently active and valid are evaluated.

Implementation note: Do not filter using Active_Exp.

Phase B - Sensor-Specific Expected Window Determine when each current sensor was expected to start transmitting.

8. Determine the experiment window start

What we do: Set rolling_start = report_now - 24 hours. If exp_started_at is inside this 24-hour window, round it upward to the next 3-minute interval. Otherwise use rolling_start.

What we get: experiment_window_start for every active experiment.

Why it matters: A new experiment is evaluated only from the time it could reasonably have begun transmitting.

9. Adjust the window for a replacement

What we do: For each current sensor, look up its Location in the replacement map. If the replacement Updated_At is inside the last 24 hours, round it upward to the next 3-minute interval and use the later of the experiment start and replacement time.

What we get: sensor_window_start for every current sensor.

Why it matters: A newly installed sensor is evaluated only from its own installation time.

Implementation note: A replacement older than 24 hours no longer changes the rolling 24-hour window.

10. Calculate expected packets

What we do: Use 480 expected packets for a full 24-hour window. For a shorter window, calculate floor((report_now - sensor_window_start) / 3 minutes) + 1, capped at 480.

What we get: Expected_Packets for each current sensor.

Why it matters: This is the correct denominator for Packet Loss and accounts for both new experiments and recent replacements.

Phase C - BigQuery Retrieval and Experiment State Retrieve raw battery packets once and use them for all downstream metrics.

11. Query BigQuery for battery data

What we do: Query the last 25 hours from iucc-f4d.Field4D.F4D_sensors_data for all Owner + Mac_Address pairs from the device inventory, with Variable = 'battery'. Do not filter by the Firestore sensor LLA list.

What we get: Raw rows containing Owner, Mac_Address, Exp_Name, LLA, Location, Timestamp, Battery Value, and Time_Zone.

Why it matters: The same query supports Packet Loss, battery analysis, and detection of recently stopped experiments.

Implementation note: The extra hour provides safe coverage; Packet Loss and battery logic still use their defined 24-hour windows.

12. Detect recently stopped experiments

What we do: Group BigQuery rows by Owner + Mac_Address + Exp_Name and compare them with the Firestore active-experiment list. If an experiment is not active in Firestore but its latest BQ Timestamp is within the last 24 hours, mark it as stopped.

What we get: A stopped-experiment result containing the experiment identity and MAX(Timestamp).

Why it matters: A recently stopped experiment is reported instead of silently disappearing.

Implementation note: Do not calculate sensor metrics for a stopped experiment. If its last data is older than 24 hours, omit it.

Phase D - Sensor Metrics Match current sensors to BigQuery and calculate Packet Loss and Last Seen.

13. Match BigQuery data to current sensors

What we do: Match rows by Owner + Mac_Address + Exp_Name + LLA.

What we get: The battery rows belonging to each exact current sensor.

Why it matters: LLA prevents data from an old replaced sensor being mixed with the current sensor at the same Location.

14. Calculate Packet Loss

What we do: Keep only rows between sensor_window_start and report_now. Because each battery packet is unique and arrives on a 3-minute schedule, Actual_Packets is the number of rows in this window. Calculate ((Expected - Actual) / Expected) × 100 and clamp it to 0-100%.

What we get: Expected_Packets, Actual_Packets, and Packet_Loss_Percentage.

Why it matters: This measures transmission continuity over the correct sensor-specific period.

Implementation note: If no rows are found, Actual_Packets = 0 and Packet Loss = 100%.

15. Read Last_Seen from Firestore

What we do: Use Last_Seen from /GCP-FS/last-package. Do not round it. Calculate report_now - Last_Seen.

What we get: The most recent Firestore contact time and its age.

Why it matters: It provides an immediate current-activity indicator independent of the 24-hour Packet Loss calculation.

16. Classify current transmission status

What we do: If Last_Seen is 15 minutes old or less, report Is Active = ✓. If it is older than 15 minutes, report Is Active = X.

What we get: A simple current transmission status for each sensor.

Why it matters: Packet Loss describes the full monitoring window, while this field shows whether the sensor is transmitting now.

Phase E - Battery Analysis Evaluate battery health using only battery packets from the last 24 hours.

17. Create the 24-hour battery dataset

What we do: From the 25-hour BigQuery result, keep only each sensor's battery rows from report_now - 24 hours through report_now.

What we get: The battery measurements eligible for battery-status analysis.

Why it matters: The extra hour used for retrieval never enters the battery calculation.

18. Handle a sensor with no recent BigQuery battery data

What we do: If the current sensor has no battery row in the last 24 hours, do not classify its battery using BigQuery. Display Last_Package.battery from Firestore instead.

What we get: A message such as: 'No data for 24H - Last battery from Firestore: 2738 mV.'

Why it matters: The last known value remains visible, but the report does not incorrectly assign OK to stale information.

19. Prepare recent battery measurements

What we do: If battery rows exist, sort them by Timestamp descending. Use all measurements when fewer than 20 exist; otherwise use only the latest 20.

What we get: N, the number of measurements used, with N capped at 20.

Why it matters: Battery status is based on the most recent available evidence within the last 24 hours.

20. Calculate low-battery statistics

What we do: Define a low measurement as Battery Value < 2700 mV. Calculate Low_Count and Low_Percentage = Low_Count / N × 100.

What we get: N, Low_Count, and Low_Percentage for every sensor with recent BQ data.

Why it matters: These values drive the battery classification rules.

21. Classify 1-3 measurements

What we do: If N is 1-3: Low_Count >= 2 → POSSIBLE BATTERY ISSUE; otherwise → INSUFFICIENT BATTERY DATA.

What we get: A cautious status for sensors with very little recent data.

Why it matters: The system can flag repeated low values without treating sparse normal values as proof of a healthy battery.

22. Classify 4-9 measurements

What we do: If N is 4-9: Low_Percentage >= 60% → REPLACE BATTERY; otherwise, Low_Count >= 2 → POSSIBLE BATTERY ISSUE; otherwise → LIMITED BATTERY DATA.

What we get: A battery status adapted to a small but meaningful sample.

Why it matters: The rule separates a dominant low-voltage pattern from isolated low readings.

23. Classify 10-19 measurements

What we do: If N is 10-19: Low_Percentage >= 50% → REPLACE BATTERY; 10% <= Low_Percentage < 50% → POSSIBLE BATTERY ISSUE; Low_Percentage < 10% → LIMITED BATTERY DATA.

What we get: A battery status based on the proportion of low measurements.

Why it matters: With more data, percentage thresholds provide a more stable decision.

24. Classify 20 measurements

What we do: When at least 20 measurements exist, analyze only the latest 20. Low_Percentage > 20% → REPLACE BATTERY; otherwise, Low_Count >= 3 → POSSIBLE BATTERY ISSUE; otherwise → OK.

What we get: The final battery status for sensors with a full recent sample.

Why it matters: OK is assigned only when sufficient recent data exists and fewer than three measurements are low.

Phase F - Final Results and Report Structure Create clear sensor and experiment outputs before applying recipient permissions.

25. Create the final sensor and stopped-experiment outputs

What we do: For each current sensor, store Owner, Mac_Address, Device_Name, Exp_Name, Location, LLA, Last_Seen, current activity, Expected Packets, Actual Packets, Packet Loss, battery statistics, and Battery Status. For a stopped experiment, store the experiment identity and last BQ Timestamp.

What we get: A complete system result that contains both active sensor status and recently stopped experiments.

Why it matters: All calculations are completed once before any email-specific filtering.

26. Build the report hierarchy and preserve source roles

What we do: Organize results as Owner → Device_Name (Mac_Address) → Experiment → Sensors. Show a stopped-experiment notice at experiment level. Keep source responsibility explicit: F4D_mac_to_device defines devices, Firestore defines expected active state, and BigQuery shows actual transmission.

What we get: A readable, traceable report structure.

Why it matters: The report remains understandable and a missing transmission never causes an active Firestore sensor or experiment to disappear.

Phase G - Permissions and Email Distribution Use permissions only after the complete system result has been calculated.

27. Load permissions and distribute completed results

What we do: Load iucc-f4d.Field4D.F4D_permissions near the beginning of the run and store Email, Owner, Mac_Address, and Experiment. After all active and stopped experiment results are complete, match each experiment to permission rows. A permission applies when Owner and Mac_Address match and Experiment equals the exact Exp_Name or '*'. Group permitted experiment results by Email and send one filtered report per recipient.

What we get: A recipient-specific report containing only the devices and experiments that the email address is allowed to view.

Why it matters: The system performs Firestore calls and BigQuery calculations only once, while permissions control delivery rather than data collection.

Implementation note: Do not use permissions to restrict the 25-hour BigQuery query. '*' means all experiments for the matching Owner + Mac_Address.

# End-to-End Flow

| # | Pipeline action |
| --- | --- |
| 1 | Set report_now once. |
| 2 | Load Owner, Mac_Address, and Device_Name from F4D_mac_to_device. |
| 3 | Load permission rules from F4D_permissions. |
| 4 | Call metadata/experiments for every device and keep active_count > 0. |
| 5 | Call last-package for every active experiment. |
| 6 | Identify replaced records and build the latest replacement-time map. |
| 7 | Keep current sensors where Is_Active = true and Is_Valid = true. |
| 8 | Calculate sensor_window_start and Expected_Packets. |
| 9 | Query 25 hours of Variable = 'battery' data for every device. |
| 10 | Detect experiments stopped within the last 24 hours. |
| 11 | Match active sensors to BQ by Owner + MAC + Experiment + LLA. |
| 12 | Calculate Packet Loss and current Last Seen status. |
| 13 | Analyze battery data from the last 24 hours and assign the battery status. |
| 14 | Create the complete system result. |
| 15 | Apply permissions and send one filtered report per email address. |

Final design principle Build the full technical status once, then use permissions only to decide who receives each completed experiment result.